Gambaran Hasil Pewarnaan Pemeriksaan Mikroskopis Feses Menggunakan Daun Pacar Air (Impatiens balsamina L) yang Direndam 6 dan 12 Jam

An Overview Of Stool Microscopic Examination Staining Results Using Water Balsam Leaf (Impatiens balsamina L) Soaked For 6 And 12 Hours

ABSTRACT

Worm infections remain a public health problem in Indonesia, particularly in areas with inadequate sanitation. Microscopic examination of stool samples using the direct slide method commonly relies on 2% eosin as a stain; however, this synthetic dye is known to have a negative environmental impact, is poorly biodegradable, flammable, and relatively expensive. Balsam leaves (Impatiens balsamina L) contain anthocyanin pigments that have potential as a more environmentally friendly natural dye alternative. This study aimed to describe the clarity of helminth egg preparations using balsam leaf extract with soaking times of 6 and 12 hours in microscopic stool examination using the direct slide method. This study employed a qualitative approach with an experimental design. The samples consisted of 15 stool specimens from waste collection workers at TPST 3R Desa Adat Mengwi, which were first examined using 2% eosin as a positive control. Specimens found positive for Soil-Transmitted Helminth (STH) eggs were re-examined using natural dye prepared by soaking balsam leaves in 96% alcohol for 6 and 12 hours, with three replications per treatment. Assessments were made of field-of-view clarity and the distinctness of egg morphology. Of the 15 samples examined with 2% eosin, 3 samples (20%) were positive for Ascaris lumbricoides eggs. Examination using balsam leaf extract at both 6-hour and 12-hour soaking times showed that all preparations achieved optimal clarity scores, with a clear field of view and well-observable egg morphology, comparable to the 2% eosin control. Balsam leaf extract can therefore be used as an alternative natural dye in microscopic stool examination, and since both soaking times produced equivalent results, a 6-hour soaking period can be selected as the more time-efficient option.

Keywords: Balsam Leaf, Natural Dye, Helminth Eggs, Soil Transmitted Helminth, Direct Slide

ABSTRAK

Infeksi kecacingan masih menjadi masalah kesehatan masyarakat di Indonesia, terutama di daerah dengan sanitasi yang kurang memadai. Pemeriksaan mikroskopis feses dengan metode direct slide umumnya menggunakan eosin 2% sebagai pewarna, namun penggunaan pewarna sintetis ini diketahui berdampak negatif terhadap lingkungan, sulit terurai, mudah terbakar, dan relatif mahal. Daun pacar air (Impatiens balsamina L) mengandung pigmen antosianin yang berpotensi digunakan sebagai pewarna alami alternatif yang lebih ramah lingkungan. Penelitian ini bertujuan untuk mengetahui gambaran kejernihan sediaan telur cacing menggunakan perasan daun pacar air dengan waktu perendaman 6 jam dan 12 jam pada pemeriksaan mikroskopis feses metode direct slide. Penelitian ini merupakan penelitian kualitatif dengan desain eksperimental. Sampel penelitian berupa 15 sediaan feses petugas pengangkut sampah di TPST 3R Desa Adat Mengwi, yang selanjutnya diperiksa menggunakan eosin 2% sebagai kontrol positif. Sampel yang dinyatakan positif mengandung telur Soil Transmitted Helminth (STH) kemudian diperiksa ulang menggunakan pewarna alami perasan daun pacar air yang direndam dalam alkohol 96% selama 6 jam dan 12 jam, dengan tiga kali pengulangan pada setiap perlakuan. Penilaian dilakukan terhadap kejernihan lapang pandang dan kejelasan morfologi telur cacing. Dari 15 sampel yang diperiksa dengan eosin 2%, ditemukan 3 sampel (20%) positif mengandung telur Ascaris lumbricoides. Hasil pemeriksaan menggunakan perasan daun pacar air pada perendaman 6 jam maupun 12 jam menunjukkan bahwa seluruh preparat memperoleh nilai kejernihan optimal (skor 2), dengan lapang pandang yang jelas dan morfologi telur cacing yang dapat diamati dengan baik, setara dengan hasil kontrol eosin 2%. Dengan demikian, perasan daun pacar air dapat digunakan sebagai pewarna alami alternatif pada pemeriksaan mikroskopis feses, dan karena kedua waktu perendaman memberikan hasil yang setara, perendaman selama 6 jam dapat dipilih sebagai kondisi yang lebih efisien dari segi waktu.

Kata kunci: Daun Pacar Air, Pewarna Alami, Telur Cacing, Soil Transmitted Helminth, Direct Slide